☀️

SOLARSCAN AI

Final Year Project — Presentation Brief

Automated Detection and Classification of Photovoltaic Panel Defects Using YOLOv8 with TFLite Mobile Integration

| Course | BSc Information Technology |
| --- | --- |
| Department | Computer Science & Informatics |
| Institution | University of Energy and Natural Resources (UENR) |
| Academic Year | 2024 / 2025 |
| Presentation Type | Final Year Project (FYP) Presentation |

# Purpose of This Document

This document serves as a comprehensive briefing for all group members preparing for the Final Year Project presentation. Every member is expected to read, understand, and be ready to confidently discuss any section. The presentation covers five required areas as specified by the department:

- Project Topic — what we built and why

- Group Members — roles and contributions

- Problem Statement — the real-world challenge we address

- Methodology — how we designed, built, and evaluated the system

- Significance — why this project matters to Ghana, Africa, and the global solar industry

⚠️ Important: All members must arrive on time and be prepared to answer questions from the panel. This brief ensures every team member understands the full scope of the project — not just their own section.

# 1. Project Topic

Automated Detection and Classification of Photovoltaic Panel Defects Using YOLOv8 with TFLite Mobile Integration

## What This Means (Plain Language)

Solar panels degrade over time due to physical defects — cracks, dust, overheating, and material failure. Currently, inspection is done manually by trained engineers walking along panel rows with thermal cameras. This is slow, expensive, and unable to scale to Ghana's growing solar capacity.

We built SolarScan AI — a mobile-first application that uses a state-of-the-art AI object detection model (YOLOv8) compressed for smartphones (TFLite) to automatically detect, classify, and report solar panel defects from a single photograph. The system works entirely on-device — no internet required — making it suitable for remote solar farms across Ghana.

## Key Technologies

| YOLOv8 | You Only Look Once v8 — the world's leading real-time object detection AI model by Ultralytics. Trained on 4,312 solar panel images across 8 defect classes. |
| --- | --- |
| TFLite INT8 | TensorFlow Lite with 8-bit integer quantisation — shrinks the model from 6.3 MB to 3.2 MB and enables <200ms inference on a mobile phone CPU without any internet or cloud connection. |
| HCI Principles | Human-Computer Interaction principles (Norman's 7 principles, Fitts' Law, WCAG 2.2 accessibility) are embedded into the mobile app design to ensure ease-of-use for field technicians. |

# 2. Group Members & Roles

The following table outlines each member's primary responsibility during the project. During the presentation, every member is expected to speak and contribute — examiners may direct questions to any individual.

| # | Full Name | Student ID | Primary Role |
| --- | --- | --- | --- |
| 1 | [Enter Full Name] | [Student ID] | Project Lead / AI Model Training |
| 2 | [Enter Full Name] | [Student ID] | Mobile App Development |
| 3 | [Enter Full Name] | [Student ID] | HCI Design & User Testing |
| 4 | [Enter Full Name] | [Student ID] | Data Collection & Documentation |

📝 Fill in your full names and student IDs before the presentation day.

# 3. Problem Statement

## The Global Solar Energy Challenge

Solar photovoltaic (PV) energy is the fastest-growing electricity source worldwide. Ghana's government has committed to achieving 10% renewable energy by 2030, with solar playing a central role. However, solar panels degrade silently — defects develop over years of outdoor exposure and are invisible to the naked eye until significant damage has occurred.

| Global Revenue Lost $10B/yr | Capacity Added (2024) 452 GW | Manual Inspection Rate 1–2 MW/day | Efficiency Loss (avg) 5–30% |
| --- | --- | --- | --- |

## Core Problems This Project Solves

- A trained technician can manually inspect only 1–2 MW of panels per day using a handheld thermal camera. A 100 MW solar farm would take months to fully inspect manually.Scale:

- Professional thermal drone surveys cost $500–$2,000 per MW, making routine inspection unaffordable for smaller solar installations in Ghana.Cost:

- Defects like Potential-Induced Degradation (PID), micro-cracks, and early-stage delamination are invisible to the human eye and undetectable without specialised imaging equipment.Invisibility:

- Unreported defects compound over time. A single hotspot cell left unrepaired can reduce an entire string's output by up to 20% and create a fire hazard.Revenue Loss:

- Remote solar farms in northern Ghana and off-grid communities have no internet access, making cloud-based AI inspection systems unusable.Connectivity:

## Specific Defects Detected by SolarScan AI

| Defect Type | Cause | Revenue Impact |
| --- | --- | --- |
| 🔥 Hotspot | Reverse current, shading | Up to 9% per affected area |
| ⚡ Micro-crack | Mechanical/thermal stress | Up to 6% loss |
| 🌫️ Soiling / Dust | Particulate accumulation | 5–30% output reduction |
| 💡 Bypass Diode Fault | Overvoltage failure | Up to 20% — fire risk |
| 🧊 Delamination | UV / moisture degradation | 7% per affected area |
| 🎨 Discoloration | EVA yellowing | 4–15% light transmission loss |
| 🐌 Snail Trail | Silver paste oxidation | 2% — indicates micro-crack |
| ⚗️ PID Degradation | Voltage leakage to ground | Up to 30% power loss |

# 4. Methodology

Our methodology follows the Double Diamond design process (Discover → Define → Develop → Deliver) combined with a standard AI/ML development pipeline. This ensures both technical rigour and user-centred design.

## Phase 1 — Discover & Define

- Conducted literature review of 12+ peer-reviewed papers on PV defect detection (2023–2025)

- Identified key real-world challenges: scale, cost, connectivity, and technician distrust of AI

- Defined 8 target defect classes based on most economically impactful defect types in tropical climates

- Created 3 user personas: Field Technician, Plant Manager, and Energy Consultant

## Phase 2 — Dataset Preparation

- Aggregated 4,312 images from 4 sources: Roboflow Solar Panels Universe (1,820), Kaggle SPCI Dataset (1,340), PVEL-AD (742), and custom field-captured thermal images (410)

- Split dataset: 72% training (3,110) / 15% validation (648) / 13% test (554)

- Applied 7 augmentation techniques: Mosaic 4-image, HSV jitter ±5%, horizontal/vertical flip, rotation ±15°, scale jitter 0.5–1.5×, Cutout patching, and Copy-paste

## Phase 3 — Model Training

- Base model: YOLOv8n (nano) — 3.01M parameters, 8.1 GFLOPs, pretrained on COCO

- Training: 120 epochs, batch size 16, image size 640×640, AdamW optimizer, lr=0.001

- Early stopping patience = 20 epochs — no overfitting detected (val_loss tracks train_loss within ±0.04)

- Best weights saved at epoch 118 based on validation mAP@50

## Phase 4 — Model Evaluation

| mAP@50 92.7% | Precision 91.8% | Recall 89.4% | F1-Score 90.6% |
| --- | --- | --- | --- |

- Confusion matrix analysis: Soiling (96.3%) and Hotspot (93.1%) achieved highest recall

- Snail Trail (83.1%) most commonly confused with Delamination — expected given morphological similarity

- Model benchmarked against YOLOv5s, YOLOv8n baseline, YOLOv11, and PV-YOLOv12n from 2025 literature

## Phase 5 — TFLite Export & Mobile Integration

- Model exported to TFLite INT8 format using post-training quantisation with calibration dataset

- Model size reduced from 6.3 MB → 3.2 MB (49% reduction); inference latency: 168ms on mobile CPU

- App functions fully offline — no internet or cloud dependency required

## Phase 6 — HCI Design & User Testing

- Applied Norman's 7 Principles, Fitts' Law, Gestalt Proximity, and WCAG 2.2 accessibility guidelines

- Designed 6-step onboarding flow, Grad-CAM XAI explanations, and Human-in-the-Loop field notes

- System Usability Scale (SUS) testing conducted with 8 participants across 5 professional roles

- Average SUS Score: 83.4 / 100 — rated 'Good' (industry average: 68.0)

- Task completion rate: 100% for core scan and XAI functions; 87.5% for export and batch scan

# 5. Significance of the Project

## National Significance — Ghana

- Ghana's Renewable Energy Master Plan targets 1,363.63 MW of solar capacity by 2030. SolarScan AI directly supports the maintenance infrastructure needed to sustain this ambition.

- The app is designed for offline use, addressing the connectivity gap in remote solar installations in northern Ghana, the Volta Region, and off-grid rural communities.

- At a cost of zero (mobile app, no subscription), SolarScan AI democratises professional-grade PV inspection previously available only to well-funded utility companies.

## Technical Significance

- First student-developed YOLOv8 + TFLite solar defect detection system with 8-class coverage benchmarked against peer-reviewed 2025 models.

- Achieves competitive mAP@50 of 92.7% while being 2× smaller than comparable models — a meaningful contribution to edge AI research for constrained devices.

- Demonstrates that INT8 quantisation can maintain >92% detection accuracy, contributing to the emerging field of sustainable AI (small, efficient models over large cloud-dependent ones).

## Economic Significance

- A 100-panel solar installation generating $18,000/year in electricity revenue loses approximately $1,620/year from undetected defects. Early detection and repair can recover 80–90% of this loss.

- Industry research (Raptor Maps, 2025) shows that AI-powered inspection yields a 3–8× return on investment compared to manual inspection costs.

- The built-in ROI Calculator in SolarScan AI allows plant managers to quantify financial risk in real time during a field inspection.

## Academic Significance

- Demonstrates mastery across multiple modules: AI/ML (YOLOv8 training), HCI (SUS testing, Norman's principles), Cloud & Modern Trends (TFLite, edge AI), and Software Development.

- All model metrics are reproducible — dataset sources, training configuration, and evaluation code are fully documented and available for replication.

- The project advances UENR's research output in applied AI for the African renewable energy sector.

## Social & Environmental Significance

- Defective solar panels that go unrepaired consume energy without generating it, wasting the embodied carbon of their manufacture. Early detection extends panel life and reduces electronic waste.

- By making inspection accessible to local technicians with a smartphone, the project builds technical capacity in Ghanaian communities that operate and maintain solar farms.

- The Human-in-the-Loop design means field corrections from technicians improve the AI model over time — creating a feedback loop that benefits the entire industry.

Presentation Day Checklist ✅ Arrive at least 10 minutes before the scheduled time ✅ All members must be present and appropriately dressed ✅ Be ready to speak on any of the 5 required sections ✅ Prepare for follow-up questions on the model, dataset, and HCI testing ✅ Know your SUS score (83.4) and what it means ✅ Know the mAP@50 (92.7%) and be able to explain it simply ✅ Know the 8 defect classes and at least one real-world consequence of each

Prepared by SolarScan AI Group  |  University of Energy and Natural Resources  |  2025